REPORT ON PROJECT PHASE I

Number plate recognition

SUBMITTED to UNIVERSITY OF PUNE

FOR PARTIAL FULFILLMENT

OF THE REQUIREMENTS for the DEGREE OF

BACHELOR OF ENGINEERING

in

Electronics and Telecommunication

By

AYUSHEE MANISH SHAH

PRACHI SATISH KELKAR

SHIVANG SHARMA

Department Of Electronics and TELECommunication

PUNE INSTITUTE OF COMPUTER TECHNOLOGY

PUNE – 43

oCTOBER 2015

Department of Electronics and Telecommunication Engineering

Pune Institute of Computer Technology, Pune – 43

CERTIFICATE

This is to certify that the Project Phase I Report entitled

“NUMBER PLATE RECOGNITION”

has been successfully completed  by

Ayushee Manish Shah

Prachi Satish Kelkar

Shivang Sharma

towards the partial fulfillment of the degree of Bachelor of Engineering in Electronics and Telecommunication as awarded by the Savitribai Phule Pune University, at Pune Institute of Computer Technology during the academic year 2015-16.

Dr. M. V. Munot					                     Prof. Dr. Y. Ravinder Project Guide								  HOD, E&TC Dept.

ACKNOWLEDGEMENTS

This project could not have been implemented without the guidance of our guide, Prof. M. V. Munot. She showed us how working as a team was important and helped tackle the problems we faced.

We would also like to thank Prof. M. Turuk for her patience with our problems in the subject of Digital Image Processing.

The panel of professors who evaluated our project progress also play an integral role in making this project what it is. For the same we would like to thank Dr. Y. Ravinder,

Prof. R. C. Jaiswal and Prof. V. Y. Deo.

Also without the facilities the college provided us, the project would have been much more difficult to execute.

Ayushee Shah

Prachi Kelkar

Shivang Sharma

CONTENTS

|  | Abstract |  | I |
| --- | --- | --- | --- |
|  | List of Acronyms |  | II |
|  | List of Symbols |  | III |
|  | List of Figures |  | IV |
| 1 | Introduction |  | 1-6 |
|  | 1.1 | Background | 1 |
|  | 1.2 | Relevance | 1 |
|  | 1.3 | Literature Survey | 2 |
|  | 1.4 | Motivation | 5 |
|  | 1.5 | Scope | 6 |
|  | 1.6 | Organization of the Report | 6 |
| 2 | Block Diagram And Theory |  | 7-11 |
|  | 2.1 | Block Diagram | 7 |
|  | 2.2 | Input Vehicle Image | 8 |
|  | 2.3 | Gray Conversion | 8 |
|  | 2.4 | License Plate Extraction | 9 |
| 3 | Segmentation |  | 12-15 |
|  | 3.1 | Boundary Extraction | 12 |
|  | 3.2 | Region Fill | 13 |
|  | 3.3 | Thinning Operation | 13 |
|  | 3.4 | Segmentation | 14 |
|  | 3.5 | Feature Based Character Recognition | 14 |
| 4 | Result |  | 16-19 |
|  | 4.1 | Data Base | 16 |
|  | 4.2 | Output | 17 |
| 5 | Conclusions |  | 20 |
| 6 | Future Plan |  | 21 |
|  | References |  | 22 |

ABSTRACT

Automatic license plate recognition is one of the most promising aspects of applying computer vision techniques towards intelligent transportation system. The image is converted to gray scale and is filtered using a 3x3 median filter. Salt and pepper noise is subtracted from the image. Using an appropriate structuring element, the image is dilated and eroded. The difference of these two gave boundaries present in the image. The boundaries are enhanced using scaling and convolution operation. Using erosion and subtraction, the unnecessary horizontal lines present in the image were eliminated. The rifts and gaps in the image are suppressed by filling them. Segmentation is performed on this image and the separated characters and numbers are marked using boxes. A Graphical User Interface was prepared for the same where all the outputs for the above mentioned operations can be observed with ease. The project further includes template matching for actual character and number recognition and an algorithm like neural networking to make it self-learning. Part of the project that is yet to be executed includes Neural Networks Block. An algorithm for license plate extraction is to be added.

I

| Abbreviations and Acronyms |  |
| --- | --- |
| ALPR | Automatic License Plate Recognition |
| LPR | License Plate Recognition |
| GUI | Graphical User Interface |

II

| List of Symbols |  |
| --- | --- |
|  | Dilation |
|  | Erosion |
| ∩ | Intersection |

III

| List of Figures |  |  |
| --- | --- | --- |
| Fig. 2.1 | Block Diagram | 7 |
| Fig. 2.2 | Structural Element | 10 |
| Fig. 2.3 | Eroded Image | 10 |
| Fig. 2.4 | Diluted Image | 11 |
| Fig. 3.1 | Boundary Extracted image | 12 |
| Fig. 3.2 | Region Filling | 13 |
| Fig. 4.1 | Ideal Image | 16 |
| Fig. 4.2 | Blurred Image | 16 |
| Fig. 4.3 | Tilted Image | 16 |
| Fig. 4.4 | Original License Plate Image | 17 |
| Fig. 4.5 | Boundary Extracted image | 17 |
| Fig. 4.6 | Extracting Dimensions | 18 |
| Fig. 4.7 | Region Filling | 18 |
| Fig. 4.8 | Character Segmentation | 19 |
| Fig. 4.9 | GUI in Matlab | 19 |

IV

| List of Tables |  |  |
| --- | --- | --- |
| Table 1.1 | IEEE papers | 3 |
| Table 1.2 | IEEE survey | 5 |